Fork assembly instructions

## Warehouse

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A |  |  |  |  |  |  |  |
| B |  |  |  |  |  |  |  |

## List of parts needed

Take the following parts from stock:

- SNP1 (quantity: 2 pcs) à from shelf A1

- GPP11 (quantity: 2 pcs) à from shelf A2

- GNP21 (quantity: 1 pc) à from shelf A2

- GNE22 (quantity: 2 pcs) à from shelf A2

- PG1 (quantity: 2 pcs) à from shelf A3

- PN3 (quantity: 2 pcs) à from shelf A3

- F1 (quantity: 2 pcs) à from shelf A3

- PON (quantity: 1 pc) à from shelf B3

- ELA (quantity: 1 pc)  from shelf B7

## Assembly

### Step 1 - Withdraw Components for PIECE 1SARA

Required pieces for creating Piece 1SARA

- SNP1SARA x 29 pcsà from shelf A1SARA

- PG1SARA x 20 pcà from shelf A3SARA

Step 2 - Assembly PIECE 1SARA

Insert the two PG1SARA pieces into the holes of the two SNP1SARA pieces.

- Position the two SNP1SARA pieces so that the two 'L' shapes are mirrored, meaning, pointing in the same direction.

- Insert the two PG1SARA pieces into the cross-shaped holes located at the ends of the SNP1SARA pieces. The two gray axes should be centered into the two L-shaped pieces.

### Step 3 - Withdraw Components for PIECE 2SARA

Required pieces for creating Piece 2SARA:

- Piece 1SARA

- GNE22 x 28 pcsà from shelf A2SARA

### Step 4 - Assembly PIECE 2SARA

Creation piece 2SARA

- Insert the upper horizontal axis of Piece 1SARA into the cross-shaped holes of the two GNE22SARA pieces.

- Fully insert the two GNE22SARA pieces at the left and right ends of the upper axis.

- The protruding parts should be facing you and parallel to the axis of the SNP1 workpiece.

### Step 5 - Withdraw Components for PIECE 3

Required pieces for creating Piece 3:

- Piece 2

- GPP11 x 2 pcsà from shelf A2

### Step 6- Assembly PIECE 3

Creation Piece 3

- Insert the lower horizontal axis of Piece 1 into the cross-shaped holes of the two GPP11 pieces.

- Place the of the two GPP11 pieces at the left and right ends of the lower axis and insert them completely.

- Ensure that the hole in the protruding part of each GPP11 piece is aligned with the SNP1 axle , so that the hole is in line with the L-shaped piece.

### Step 7 - Withdraw Components for PIECE 4

Required pieces for creating Piece 4:

- PN3 x 2 pcsà from shelf A3

- GNP21 x 1 pcà from shelf A2

### Step 8 - Assembly PIECE 4

Creation Piece 4

1. Insert the two PN3 pieces into the holes of GNP21.

- Fully insert the two PN3 pieces into the two holes of GNP21. Once fitted, the 2 PN3 pieces should be parallel to each other and perpendicular to the GNP21 piece.

### Step 9 - Withdraw Components for PIECE 5

Required pieces for creating  Piece 5:

- PON x 1 pcsà from shelf B3

- Piece 4

### Step 10 - Assembly PIECE 5

Creation Piece 5

1. Insert PON into Piece 4.

- Insert PON completely into the central hole of Piece 4 so that only the round part protrudes.

- Ensure that PON is perpendicular to Piece 4.

### Step 11 - Assembly PIECE 6

Required pieces for creating Piece 6:

- Piece 3

- Piece 5

Creation piece 6

1. Join Piece 3 with Piece 5.

- Fully insert the two long sides (PN3) of Piece 5 into the remaining cross-shaped holes of Piece 3.

### Step 12 - Withdraw Components for PIECE 7

Required pieces for creating Piece 7:

- Piece 6

- ELA x 1 pcà from shelf B7

### Step 13 - Assembly PIECE 7

Creation piece 7

1. Attach the ELA elastic to Piece 6.

- Stretch the elastic ELA by fixing one end to the round part (PON) of Piece 6 and extending it to the low part of Piece 3.

- Fix the elastic ELA around the two pieces in the center (GPP11) and extend it to the bottom of the L-shaped part of Piece 3.

### Step 14 - Withdraw Components for PIECE 8

Required pieces for creating Piece 8:

- F1 x 2 pcà from shelf A3

- Piece 7

### Step 15 - Assembly PIECE 8

Creation piece 8

1. Insert the two F1 pieces into Piece 7.

- Fully insert the two F1 pieces into the front holes (GPP11) of Piece 7.

- Ensure to insert the side of the axis that has a small protrusion.

- The two F1 pieces should be perpendicular to piece 7.

### Step 16 – Final Control/ Quality control

Control test

- Push down the part where the two gray axes (F1) are inserted and check If the movement creates tension in the elastic ELA.

- If the elastic does not create enough tension, make sure that all parts are properly assembled and that the elastic is not loose.